# MicroHydroV1 – Tank Ripple Test Procedure

Document Type: Engineering Verification Test

Test Name: Ripple Attenuation Performance Test

| Version | 1.0 |
| --- | --- |
| Date | __________________ |
| Prepared by | __________________ |
| Approved by | __________________ |

## 1. Objective

Evaluate the ability of the MicroHydroV1 inlet system to reduce free‑surface ripple amplitude in a controlled water tank environment.

Performance Requirement: Ripple amplitude reduction of −30% to −50% compared to a baseline straight inlet under the same flow conditions.

## 2. Equipment and Materials

| Item | Description / Notes |
| --- | --- |
| Water Tank | Sufficient length to allow inlet‑induced surface disturbance to develop (≥ 1.0 m recommended). |
| MicroHydroV1 Test Article | Complete inlet assembly mounted per PT mounting sketch. |
| Baseline Straight Inlet | Same diameter and flow capacity as MicroHydroV1 inlet. |
| Pressure Transducer (PT) | Mounted per PT mounting sketch; suitable for low‑frequency free‑surface fluctuation measurement. |
| DAQ / Logger | Minimum sampling frequency: 50–200 Hz. Must support PT calibration constants. |
| Flow Source | Pump or gravity feed with controllable flow rates. |
| Markers | Rulers/adhesive markers on tank wall for visual reference. |
| Stopwatch | Timing steady‑state intervals. |
| Calibration Sheet | PT calibration constants and zero‑offset reference. |

## 3. Test Setup

### 3.1 Mechanical Setup

Install the MicroHydroV1 inlet in the tank using the designated fixture.

Install the Pressure Transducer (PT) per the PT mounting sketch: diaphragm downward into water; fixed depth 5–20 mm below nominal surface; provide strain relief.

Place visual markers along the tank wall at known heights for optical verification.

### 3.2 Electrical Setup

Connect PT to DAQ/logger using shielded cabling.

Enter PT calibration constants (sensitivity and offset).

Verify grounding to minimize noise.

### 3.3 Baseline Conditions

| Water depth | __________________ |
| --- | --- |
| Temperature | __________________ |
| Flow rate range | __________________ |
| Ambient lighting | __________________ |

## 4. Test Procedure (Step-by-Step)

### 4.1 Pre‑Test Checks

Confirm tank is filled to nominal depth.

Verify PT zero reading out of water.

Submerge PT to test depth; confirm stable reading.

Verify DAQ is recording at chosen sampling rate.

### 4.2 Baseline Straight Inlet Test

Install the straight inlet.

Set flow source to Flow Rate 1 and allow 30–60 s to stabilize.

Record ≥ 20 s of PT data.

Repeat for all planned flow rates (FR1…FRn).

Save files as: Baseline_FR1, Baseline_FR2, …

### 4.3 MicroHydroV1 Ripple Test

Replace straight inlet with MicroHydroV1 inlet assembly.

Repeat Section 4.2 for identical flow rates.

Save files as: MHV1_FR1, MHV1_FR2, …

### 4.4 Optional Visual Observation

Place camera orthogonal to tank wall markers.

Record 10–20 s of footage per flow rate.

Note any vortexing, sloshing, or asymmetric disturbances.

### 4.5 Post‑Test Tasks

Perform PT zero‑check again to detect drift.

Back up all datasets to repository.

Drain or secure test facility.

## 5. Data Intake Fields

### 5.1 Test Metadata

| Test operator | __________________ |
| --- | --- |
| Date | __________________ |
| Test article version | __________________ |
| PT serial number | __________________ |
| DAQ sampling rate | __________________ |
| Water depth | __________________ |

### 5.2 Flow Conditions

| Flow Rate ID | Target Flow (L/min) | Actual Flow (L/min) | Notes |
| --- | --- | --- | --- |
| FR1 |  |  |  |
| FR2 |  |  |  |
| FR3 |  |  |  |

### 5.3 PT Data Files

| Condition | File Name | Duration (s) | Comments |
| --- | --- | --- | --- |
| Baseline FR1 |  |  |  |
| Baseline FR2 |  |  |  |
| MHV1 FR1 |  |  |  |
| MHV1 FR2 |  |  |  |

### 5.4 Derived Values

| Metric | Baseline | MicroHydroV1 | % Change |
| --- | --- | --- | --- |
| RMS Ripple Amplitude (Pa or mm) |  |  |  |
| Peak‑to‑Peak Ripple |  |  |  |
| Dominant Frequency (Hz) |  |  |  |

Percentage Ripple Reduction Formula: %Reduction = (Ripple_baseline − Ripple_MHV1) / Ripple_baseline × 100%

## 6. Acceptance Criteria

Pass / Fail Requirements: For every tested flow rate, MicroHydroV1 must achieve 30% to 50% reduction in RMS ripple amplitude relative to baseline.

| Additional Observations | Requirement |
| --- | --- |
| Vortex formation | Not allowed |
| Asymmetric oscillations | Not allowed |
| Sudden pressure spikes | ≤ 10% of mean |

## 7. How to Interpret Results

Verify data quality: check for drift, spikes, and consistent noise floor. Apply low‑pass filtering (10–20 Hz) if necessary and documented.

Extract metrics: RMS ripple amplitude (primary), Peak‑to‑Peak amplitude (worst‑case), Dominant frequency (stability indicator).

Compute %Reduction = (Baseline − MHV1) / Baseline × 100%. Positive values indicate improvement; negative values indicate degradation.

Compare against acceptance criteria: 30%–50% reduction is PASS. <30% or >50% is FAIL (investigate under‑ or over‑damping).

Assess qualitative indicators: no vortexing, no asymmetric oscillations, no spikes >10% of mean, stable dominant frequency.

Evaluate across flow rates: MicroHydroV1 must pass at each tested flow condition.

## 8. Examples of Result Interpretations

PASS (Within Target)

Baseline RMS 12.4 mm; MHV1 RMS 7.1 mm → 42.7% reduction. No vortexing; stable 1.8 Hz dominant frequency. PASS.

FAIL (Insufficient Reduction)

Baseline RMS 9.8 mm; MHV1 RMS 8.9 mm → 9.2% reduction. Asymmetric sloshing and spikes observed. FAIL; check alignment and pump transients.

FAIL (Over‑Reduction)

Baseline RMS 10.2 mm; MHV1 RMS 3.8 mm → 62.7% reduction (>50%). Possible flow choking or PT depth error. FAIL.

FAIL (Worse Than Baseline)

Baseline RMS 6.4 mm; MHV1 RMS 7.2 mm → −12.5% (degradation). Vortex observed; redesign or setup correction required.

Borderline – Engineering Judgment

Baseline RMS 14.0 mm; MHV1 RMS 10.1 mm → 27.8% (below 30%). Stable behavior; repeat run to confirm before concluding FAIL.

## 9. Glossary of Terms

| Term | Definition |
| --- | --- |
| Baseline Inlet (Straight Inlet) | Reference tube geometry establishing uncontrolled ripple condition. |
| DAQ (Data Acquisition System) | Hardware that samples/logs sensor outputs (50–200 Hz sampling for this test). |
| Dominant Frequency | Main oscillation frequency in the ripple spectrum. |
| Flow Rate | Volumetric inflow to the tank (L/min), held constant between comparisons. |
| Free‑Surface Ripple | Surface undulations induced by inlet disturbances/turbulence. |
| MicroHydroV1 Inlet | Test article designed to reduce ripple amplitude. |
| Peak‑to‑Peak (P‑P) Amplitude | Max minus min within a time window. |
| Pressure Transducer (PT) | Sensor converting pressure fluctuations to electrical signal. |
| PT Calibration Constants | Sensitivity and offset used to convert voltage to pressure units. |
| Ripple Amplitude (RMS) | Root‑mean‑square pressure fluctuation magnitude. |
| Ripple Reduction Percentage | (Baseline − MHV1)/Baseline × 100%. |
| Steady‑State Condition | Period after a change when flow and ripple are consistent. |
| Vortex Formation | Undesirable swirling flow increasing disturbance. |

## 10. Results Summary

| Flow Rate | Reduction Achieved (%) | Pass/Fail | Notes |
| --- | --- | --- | --- |
| FR1 |  |  |  |
| FR2 |  |  |  |
| FR3 |  |  |  |

## 11. Signoff

| Test Engineer | __________________ Date: __________ |
| --- | --- |
| Engineering Reviewer | __________________ Date: __________ |

## Appendix A – Visual Examples of Waveforms

A1. PASS case: MicroHydroV1 ~40–45% reduction vs Baseline; similar dominant frequency; visibly lower amplitude.

A2. Over‑damped case (>50% reduction): unusually flat waveform and frequency shift. Investigate choking/PT depth.

A3. Worse‑than‑baseline case: higher amplitude and irregular peaks; likely vortexing or setup issue.