# MicroHydroV1 — Bench Session Run-of-Show

Generated: 2026-01-21 11:11 UTC

# Pre-Run Checks

Safety & spill containment

Instruments calibrated (camera, PT, wattmeter)

ELC narrow window; nominal load ready

Open RnD workbook → Measurements

# T-001 — Jet Coherence

Stabilize head

Capture images/video

Compute coherence; log Value (Sensor="coherence")

Pass: ≥ 0.90

# T-002 — Tank Ripple

Record baseline ripple

Run hyperbolic inlet and record PT

Log Baseline_Value & Value (Sensor="ripple")

Pass: ≤ 0.70 × Baseline; no entrainment increase

# T-003 — Electrical Stability

Apply nominal load; observe ELC

Log drift Hz (Sensor has "ELC"/"stability")

Pass: ≤ 0.1 Hz; no hunting

# T-004 — Power Output

Measure power (kW) at operating head

Log Value (Sensor="power")

Pass: 10–12 kW

# Post-Run

Save CSVs; run import_measurements.py

Review Dashboard & Trends; capture screenshots

Update Stakeholder deck