# Volume B3 — Microhydro (Off‑Grid Engineering)

NREL’s small hydropower fact sheet defines microhydro as systems up to 100 kW and notes that a ~10 kW system can power a large home or small farm/resort, depending on load. citeturn1search17

DOE’s Microhydropower Handbook provides extensive engineering guidance for microhydro components and design considerations. citeturn1search16

## Core equations

Approximate mechanical power:

P ≈ ρ g Q H η

ρ ≈ 1000 kg/m³, g ≈ 9.81 m/s²

Q = flow (m³/s)

H = net head (m)

η = overall efficiency (turbine+generator+drive+losses)

## Practical design steps

1) Measure gross head

2) Estimate design flow and seasonal minimum

3) Calculate penstock losses and net head

4) Select turbine class (impulse vs reaction) and generator

5) Design intake, screening, trash rack, and sediment handling

6) Electrical integration (dump load / inverter / battery / grid‑forming)

ATTRA provides a beginner‑oriented but practical overview of micro‑hydro project stages, including regulatory and equipment considerations. citeturn1search19